SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: Semiconductor Devices		COURSE CODE: ECA01

COURSE OUTCOME COVERED

CO1-Apply the principles of semiconductor physics to analyze the operation, characteristics, and applications of PN junction diodes, special diodes, and optoelectronic devices using simulation tools (BL3,BL4)

Assessment Tool Weightage	: 40%

Assessment Tool 			: Analytical Based Task

Duration				:1 Hrs

Total Marks				:50

:

Assessment will be conducted by requiring all students to answer a total of five questions. Students must carefully read each question and provide well-structured responses within the allotted time, demonstrating clarity and understanding of the subject matter.

Dr:

| Q.No | Question Bank | BL | Marks |
| --- | --- | --- | --- |
| 1. | The mobility of free electrons and holes in pure germanium are 3800 and 1800 cm2/V-s respectively. The corresponding values for pure silicon are 1300 and 500 cm2 /V-s, respectively. Determine the values of intrinsic conductivity for both germanium and silicon. Assume ni = 2.5 × 1013 cm–3 for germanium and ni = 1.5 × 1010 cm–3 for silicon at room temperature. | BL4 | 10 |
| 2. | Find the conductivity of silicon (a) in intrinsic condition at a room temperature of 300 K, (b) with donor impurity of 1 in 108 , (c) with acceptor impurity of 1 in 5 × 107 , and (d) with both the above impurities present simultaneously. Given that ni for silicon at 300 K is 1.5 × 1010 cm–3, μn = 1300 cm2 /V-s, μp = 500 cm2 /V-s, number of Si atoms per cm3 = 5 × 1022. | BL4 | 10 |
| 3. | Find the concentration (densities) of holes and electrons in N-type Silicon at 300 K, if the conductivity is 300 S/cm. Also find these values for P-type silicon. Given that for Silicon at 300 K, ni = 1.5 × 1010/cm3 , μn = 1300 cm2 /V-s and μp = 500 cm2 /V-s. | BL4 | 10 |
| 4. | In an N-type semiconductor, the Fermi level is 0.3 eV below the conduction level at a room temperature of 300 K. If the temperature is increased to 360 K, determine the new position of the Fermi level. | BL4 | 10 |
| 5. | In a P-type semiconductor, the Fermi level lies 0.4 eV above the valence band. Determine the new position of Fermi level if the concentration of acceptor atoms is multiplied by a factor of (a) 0.5, and (b) 4. Assume kT = 0.025 eV. | BL4 | 10 |
| 6 | The diode current is 0.6 mA when the applied voltage is 400 mV, and 20 mA when the applied voltage is 500 mV. Determine h. Assume kT/ q = 25 mV | BL3 | 10 |
| 7 | The reverse saturation current of the Ge transistor is 2 mA at room temperature of 25°C and increases by a factor of 2 for each temperature increase of 10°C. Find the reverse saturation current of the transistor at a temperature of 75°C. | BL3 | 10 |
| 8 | Determine the forward resistance of a PN junction diode when the forward current is 5 mA at T = 300 K. Assume silicon diode. | BL3 | 10 |
| 9 | Calculate the dynamic forward and reverse resistance of a PN junction diode when the applied voltage is 0.25 V at T = 300 K given Io = 2 mA. | BL3 | 10 |
| 10 | A PN-junction diode has a reverse saturation current of 30 mA at a temperature, of 125°C. At the same temperature, find the dynamic resistance for 0.2 V bias in forward and reverse directions. | BL3 | 10 |
| 11 | If two similar Germanium diodes are connected back to back and the voltage V is impressed upon, calculate the voltage across each diode and current through each diode. Assume similar value of Io = 1 mA for both the diodes and h = 1. | BL4 | 10 |
| 12 | (a) The resistivities of the P-region and N-region of a germanium diode are 6 W-cm and 4Ωcm, respectively. Calculate the contact potential Vo and potential energy barrier Eo. (b) If the doping densities of both P and N-regions are doubled, determine Vo and Eo. Given that q = 1.602 × 10–19 C, ni = 2.5 × 1013/cm3 , μp = 1800 cm2 /V-s, μn = 3800 cm2 /V-s and VT = 0.026 V at 300 K. | BL4 | 10 |
| 13 | In a Zener regulator, the d.c. input is 10 V ± 20%. The output requirements are 5 V, 20 mA. Assume Iz(min) and Iz(max) as 5 mA and 80 mA respectively. Design the Zener regulator. | BL5 | 10 |
| 14 | Design a Zener voltage regulator to meet the following specifications: Output voltage = 5 V, Load current = 10 mA, Zener wattage = 400 mW and Input voltage = 10 V ± 2 V. | BL5 | 10 |
| 15 | A Zener voltage regulator circuit is to maintain constant voltage at 60 V, over a current range from 5 to 50 mA. The input supply voltage is 200 V. Determine the value of resistance R to be connected in the circuit, for voltage regulation from load current IL = 0 mA to IL max, the maximum possible value of IL. What is the value of IL max? | BL5 | 10 |
| 16 | In an SCR half wave rectifier, the forward breakdown voltage of SCR is 110 V for a gate current of 1 mA. If a 50 Hz sinusoidal voltage of 220 V peak is applied, find firing angle, conduction angle, average voltage, average current, power output and the time during which SCR remains OFF. Assume load resistance is 100 Ω and the holding current to be zero. | BL5 | 10 |
| 17 | A full wave controlled rectifier employs 2 SCRs and 2 diodes in bridge configuration to rectify 230 V, 50 Hz a.c. mains and give an output of 150 V to a resistive load of 10Ω. Find the firing angle, the time during which the SCR remains OFF and the load current. | BL5 | 10 |
| 18 | In a half-wave rectifier, an a.c. voltage of peak value 24 V is connected in series with a silicon diode and load resistance of 480Ω. If the forward resistance of the diode is 20 Ω, find the peak current flowing through the diode. | BL3 | 10 |
| 19 |  |  | 10 |
| 20 |  |  | 10 |

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION FOR Q1:

| Criteria | Weightage | Level 3 (Exemplary) | Level 2 (Proficient) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- |
| Problem Understanding | 30% | Clearly interprets problem, identifies all parameters and requirements accurately (3) | Understands problem with minor gaps (2) | Misinterprets or unable to understand problem(1) |
| Solution Process | 45% | Logical, systematic steps with correct approach throughout(4.5) | Mostly correct steps with minor mistakes (4.5-3) | No clear process (2-1) |
| Result | 25% | Clearly interprets results with units and engineering relevance (2.5) | Basic interpretation with minor omissions (2.5-1.5) | No interpretation (1) |

RUBRICS FOR CALCULATION FOR Q2:

| Criteria | Weightage | Level 3 (Exemplary) | Level 2 (Proficient) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- |
| Problem Understanding | 30% | Clearly interprets problem, identifies all parameters and requirements accurately (3) | Understands problem with minor gaps (2) | Misinterprets or unable to understand problem(1) |
| Solution Process | 45% | Logical, systematic steps with correct approach throughout(4.5) | Mostly correct steps with minor mistakes (4.5-3) | No clear process (2-1) |
| Result | 25% | Clearly interprets results with units and engineering relevance (2.5) | Basic interpretation with minor omissions (2.5-1.5) | No interpretation (1) |

RUBRICS FOR CALCULATION FOR Q3:

| Criteria | Weightage | Level 3 (Exemplary) | Level 2 (Proficient) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- |
| Problem Understanding | 30% | Clearly interprets problem, identifies all parameters and requirements accurately (3) | Understands problem with minor gaps (2) | Misinterprets or unable to understand problem(1) |
| Solution Process | 45% | Logical, systematic steps with correct approach throughout(4.5) | Mostly correct steps with minor mistakes (4.5-3) | No clear process (2-1) |
| Result | 25% | Clearly interprets results with units and engineering relevance (2.5) | Basic interpretation with minor omissions (2.5-1.5) | No interpretation (1) |

RUBRICS FOR CALCULATION FOR Q4:

| Criteria | Weightage | Level 3 (Exemplary) | Level 2 (Proficient) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- |
| Problem Understanding | 30% | Clearly interprets problem, identifies all parameters and requirements accurately (3) | Understands problem with minor gaps (2) | Misinterprets or unable to understand problem(1) |
| Solution Process | 45% | Logical, systematic steps with correct approach throughout(4.5) | Mostly correct steps with minor mistakes (4.5-3) | No clear process (2-1) |
| Result | 25% | Clearly interprets results with units and engineering relevance (2.5) | Basic interpretation with minor omissions (2.5-1.5) | No interpretation (1) |

RUBRICS FOR CALCULATION FOR Q5:

| Criteria | Weightage | Level 3 (Exemplary) | Level 2 (Proficient) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- |
| Problem Understanding | 30% | Clearly interprets problem, identifies all parameters and requirements accurately (3) | Understands problem with minor gaps (2) | Misinterprets or unable to understand problem(1) |
| Solution Process | 45% | Logical, systematic steps with correct approach throughout(4.5) | Mostly correct steps with minor mistakes (4.5-3) | No clear process (2-1) |
| Result | 25% | Clearly interprets results with units and engineering relevance (2.5) | Basic interpretation with minor omissions (2.5-1.5) | No interpretation (1) |